120A Homework 2

2.24: F = a’d’ + a’c + b’cd’ + cd

a. a, b, c, d

b. a’, d’, a’, c, b’, c, d’, c, d

c. a’d’, a’c, b’cd’, cd

2.26: S = small; H = heavy; E = expensive

a. Deliver = SE’ + S’E

b. Loaded = SH’ + SH + S’H’ = S + H’

c. Packages = (SE’ + S’E) * (S + H’) = SSE’ + SE’H’ + SS’E + S’H’E = SE’ + 0 + H’SE’ + H’S’E = (1+H’)SE’ + H’S’E = SE’ + S’EH’

You can deliver small, inexpensive packages, or large expensive light packages

2.29:

F = abc + a’b

F’ = (abc + a’b)’

F’ = (abc)’(a’b)’

F’ = (a’ + b’ + c’)(a’’ + b’)

F’ = (a’ + b’ + c’)(a + b’)

F’ = a(a’ + b’ + c’) + b’(a’ + b’ + c’)

F’ = 0 + ab’ + ac’ + b’a’ + b’  + b’c’

F’ = (a+a’)b’ + b’ + ac’ + b’c’

F’ = b’ + ac’

2.32:

F = (ab’ + b)’

2.39:

F = a’b’c’ + a’bc’ + ab’c’ + ab’c + abc’

2.40:

F = a’b’c’ + a’bc’ + ab’c’ + ab’c + abc’

F = a’c’ + ab’ + abc’

F = a’c’ + a(b’ + bc’)

2.44:

| a | b | c | ab’ + b | a’c | G |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 | 1 | 1 |
| 0 | 1 | 0 | 1 | 0 | 1 |
| 0 | 1 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 1 | 0 | 1 |
| 1 | 0 | 1 | 1 | 0 | 1 |
| 1 | 1 | 0 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 | 0 | 1 |

2.47:

2.48:

a. F (a, b, c)’ = a’bc + ab = a’bc + abc’ + abc

b. F (a, b, c) = a’b = a’bc + a’bc’

c. F (a, b, c) = abc + ab + a + b + c = a’b’c + a’bc’ + a’bc + ab’c’ + ab’c + abc’ + abc

d. F (a, b, c) = c’ = abc’ + a’b’c’ + a’bc’ + ab’c’

2.52:

a.

F = ab + cd

G = (1*(ab)’(cd)’)’

F = a’b’cd + a’bcd + ab’cd + abc’d’ + abc’d + abcd’ + abcd

G = a’b’c’d’ + a’b’cd’ + a’bc’d + a’bcd’ + ab’c’d’ + ab’c’d + ab’cd’

F and G are not equivalent.

b.

| a | b | c | d | F |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 1 | 0 |
| 0 | 0 | 1 | 0 | 0 |
| 0 | 0 | 1 | 1 | 1 |
| 0 | 1 | 0 | 0 | 0 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 0 |
| 0 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 0 |
| 1 | 0 | 1 | 1 | 1 |
| 1 | 1 | 0 | 0 | 1 |
| 1 | 1 | 0 | 1 | 1 |
| 1 | 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 |

| a | b | c | d | G |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 1 |
| 0 | 0 | 0 | 1 | 1 |
| 0 | 0 | 1 | 0 | 1 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 0 | 1 |
| 0 | 1 | 0 | 1 | 1 |
| 0 | 1 | 1 | 0 | 1 |
| 0 | 1 | 1 | 1 | 0 |
| 1 | 0 | 0 | 0 | 1 |
| 1 | 0 | 0 | 1 | 1 |
| 1 | 0 | 1 | 0 | 1 |
| 1 | 0 | 1 | 1 | 0 |
| 1 | 1 | 0 | 0 | 0 |
| 1 | 1 | 0 | 1 | 0 |
| 1 | 1 | 1 | 0 | 0 |
| 1 | 1 | 1 | 1 | 0 |

The truth tables are not equal.

2.57:

Step 1: Capture Function

| M3 | M2 | M1 | M0 | C |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 1 | 0 |
| 0 | 0 | 1 | 0 | 0 |
| 0 | 0 | 1 | 1 | 1 |
| 0 | 1 | 0 | 0 | 0 |
| 0 | 1 | 0 | 1 | 1 |
| 0 | 1 | 1 | 0 | 1 |
| 0 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 1 | 1 |
| 1 | 0 | 1 | 0 | 1 |
| 1 | 0 | 1 | 1 | 1 |
| 1 | 1 | 0 | 0 | 1 |
| 1 | 1 | 0 | 1 | 1 |
| 1 | 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 |

Step 2: Create Equation

C = M3’M2’M1’M0’ + M3’M2’M1’M0 + M3’M2’M1M0’ + M3’M2M1’M0’ + M3M2’M1’M0’

Step 3: Logic Circuit

2.60:

Inverse function will output 1 if the input is greater than or equal to 16. For a 5-bit number, if there is a 1 in the left-most bit, then it is greater than or equal to 16. This will be B4. B4 will be 0 if there is any number less than 16. The equation is then: F’ = B4

Logic Circuit:

2.66:

2.75:

2.77:

a.

The path from c to F has a delay of 0.5 + 0.75 + 0.5 + 1 + 0.5 = 3.25 ns

The path from h to F has a delay of 0.5 + 1 + 0.5 + 1 + 0.5 = 3.5 ns

The path from p to F has a delay of 0.5 + 1 + 0.5 + 1 + 0.5 = 3.5 ns

The longest path is 3.5 ns. Therefore, the circuit’s critical path is 3.5 ns

b.

The path from a to F has delay of 0.5 + 1 + 0.5 + 0.75 + 0.5 + 1 + 0.5 = 4.75 ns

The path from b to F has delay of 0.5 + 1 + 0.5 + 0.75 + 0.5 + 1 + 0.5 = 4.75 ns

The path from c to F has delay of 0.5 + 0.75 + 0.5 + 1 + 0.5 = 3.25 ns

Since 4.75 ns is the longest path, that is the circuit’s critical path.